GreenHouseWatch

A Local IoT Digital Repository for Smart Greenhouse Monitoring

Problem statement

Small greenhouse operators lose plants when temperature, humidity, or soil moisture drift outside a narrow safe band — often overnight or at mid-day heat — with no local record of what happened. Commercial IoT platforms are paid and cloud-locked. This project asks how we can capture, store, and inspect greenhouse sensor data locally using only free tools so out-of-range conditions are visible instead of silent.

Use case

GreenHouseWatch simulates node GH-SENSOR-01 (air temperature, humidity, soil moisture, light). Every two seconds the node publishes JSON over MQTT to the free HiveMQ public broker (broker.hivemq.com:1883). A Python subscriber writes each payload into a local SQLite repository (iot_data.db). Matplotlib and an optional Streamlit dashboard turn that repository into time-series charts and an alert summary. The simulator follows a day/night solar curve, a slow soil-drydown, an irrigation recovery, and a few injected anomalies — the same sensor → bus → store → dashboard pattern used in smart agriculture.

Data fields

| Field | Example | Role |
| --- | --- | --- |
| timestamp, iso_time | 1788741081.58 / 2026-09-07T12:46:21Z | When the reading was taken |
| device_id | GH-SENSOR-01 | Which node sent it |
| temperature_c | 28.13 | Air temperature (°C) |
| humidity_pct | 56.07 | Relative humidity (%) |
| soil_moisture_pct | 21.56 | Root-zone moisture (%) |
| light_lux | 7693.4 | Day / night indicator |
| alert_flag, alert_reason | 1 / dry_soil | Threshold breach + cause |

System architecture

Sensor Simulator → MQTT Broker (HiveMQ) → Subscriber → SQLite → Matplotlib / Streamlit. Tools (all free): Python, paho-mqtt, broker.hivemq.com, SQLite, Matplotlib, Streamlit. Safe band: 16–32 °C, humidity 40–80 %, soil ≥ 30 %.